# Drone Swarm Project — Status Report

Internship

July 2026

# Context

The internship goal is a swarm of drones that fly in formation, following each other, without colliding, and able to keep working automatically if a drone (including the leader) disconnects.

The simulation stack is Gazebo Sim + PX4 SITL, with drone-to-drone communication built directly on MAVSDK/UDP.

# Work Done So Far

Communication and leader election between 3 drones, simulated in Gazebo with PX4 SITL:

- Each drone reads its own real GPS position from PX4 (via MAVSDK’s Telemetry plugin).

- Each drone broadcasts its position to the other two, 5 times per second, over a custom UDP protocol (36-byte packets, built with raw POSIX sockets).

- Each drone tracks the others and marks one as disconnected after 3 seconds of silence.

- Leader election rule: the leader is always the lowest surviving drone ID. Every drone recomputes this on its own every 500ms, so all drones agree without needing to coordinate.

- A simulated 6-meter radio range was added (based on real GPS distance) so a disconnection can be tested by moving a drone away, not just by killing its process.

- Fixed a “split-brain” bug: an isolated drone used to keep thinking it was the leader forever, even after reconnecting. The election is now re-evaluated continuously, so leadership correctly hands back to the rightful leader once it’s reachable again.

- Made the Gazebo world persistent, so resetting the simulation doesn’t delete the drones.

- Project is version-controlled and pushed to a private GitHub repository, with a launch_all.sh script that starts the whole simulation (Gazebo + PX4 + the 3 drone programs) in one command.

Tested and confirmed working:

- All 3 drones detect each other correctly.

- Killing the leader (drone_0) makes the other two detect the loss (~3s) and correctly elect a new leader.

- Moving a drone out of radio range and back correctly triggers disconnection and reconnection, with the leader role handed back correctly.

# New Instructions / Direction (received this week)

The supervisor gave updated guidance:

- Learn ROS 2. Communication is currently done with a custom UDP protocol; ROS 2 needs to be researched as the framework to move toward.

- Final goal clarified: the drones must be able to follow each other in formation without colliding — not just hold a logical leader/follower role, but actually fly accordingly.

- New leader re-election rule: if the leader disconnects, a new leader is elected as before. But now, when the old leader reconnects, it does not take back leadership — instead, it rejoins the swarm in the position of the drone whose place it takes (i.e., it comes back as a regular follower, not as leader again). This is a change from the current behavior, where the lowest-ID drone always wins back leadership as soon as it’s seen again.

# What’s Left To Do

- Research ROS 2 and evaluate moving the drone-to-drone communication layer onto it.

- Update the leader election logic so a reconnecting former leader rejoins as a follower instead of automatically reclaiming leadership.

- Implement actual flight formation (drones physically following the leader) using MAVSDK offboard control, building on the earlier Webots V-formation logic.

- Add collision avoidance between drones while following/flying in formation.

- Explicitly test a permanent network split (two groups that never reconnect, each keeping its own leader) — expected to work already, but not yet directly tested.